AMD-Flash Programming with BD32

The Background Interface which is part of all devices that contain a CPU32 (e.g. MC68331/332 ..... MC68360) allowes to get a system running even without any software in its memories.

It allowes also to program and erase Flash memories or EEPROMs within the system by only using the background interface. It is even possible to program EPROMs.

BD32 gives the ability to add additional commands by doing "Target resident drivers". These are programs running in the target, but can make use of the PC (keyboard, screen, disks, printer, ...)

It is not a problem, to write a "Target resident driver", which can program and erase flash memories.

The commands "AMDP8, AMDP16, AMDERA8, AMDERA16" can program and erase AMD-Flash devices Am29F010.

Each command has to be within the BD32 (or actual) directory, where the filename must be: <command>.D32. Therefore the command AMDP8 is in the file "AMDP8.D32".

Also the file <command>.MSG should be present in the same folder. This file contains the error messages. If this file is not available, only an error number is displayed without any text.

----------------------------------------------------------------------

The commands:

AMDP8 is the programming routine for 8-Bit Flash memories. Therefore if a system is using only one memory (in 8-Bit mode), this command has to be used.

AMDP16 is the programming routine for 16-Bit Flash memories. Therefore if a system is using two 8-Bit memories (giving a 16-Bit device for the CPU), this command has to be used. For this command there is one important limitation: Normally the hardware gives only one chip-select and only one write-strobe for this configuration. Therefore it is not possible to program a single byte. Always a word must be programmed. This can only be done, if the S-Record which contains the data consists of "Word"-records; this means that each line of the S-Record file must contain an even number of data bytes, otherwise AMDP16 generates an error. The number of data bytes for each line in an S-Record can normally be selected by the program, which creates the S-Record (e.g. the linker).

If this is not possible, there is a program "SREC" within the Motorola Munich mailbox, which is able to convert a file. See the description of this program for further information.

AMDERA8 is the erase routine for 8-Bit Flashes (This commands belongs to AMDP8).

AMDERA16 is the erase routine for 16-Bit Flashes (This commands belongs to AMDP16).

----------------------------------------------------------------------

Preparation for programming:

Before programming can take place, several initilization steps has to be done in the target system (e.g. switching off the software watchdog, defining the Chip-Selects with the desired address space, defining Output-Pins in a way to not affect the system, ...)

These steps have to be done before programming or erasing can be started. These steps are normally done with "MM" (memory modify) commands. For this the macro utility "Do-Files" of BD32 can be used efficiently.

Also before programming can be started, there must be at least 2kByte RAM present in the system. For example the internal RAM of the 332 can be used for this. This RAM is required for the program, for data and the stack.

After these steps are done, programming can start.

Example:

The File "AMD.DO" showes an example for these initialization steps, if the target flash should be on address 0, the CE-Input of the flash is grounded, OE of the flash is connected to CSBOOT of the 68332 and PGM of the flash is connected to CS0 of the 68332 .

----------------------------------------------------------------------

The syntax for programming / erasing is:

AMDP8 <filename> <start_of_flash>

The parameter <filename> ist the name of the S-Record file, including its path, if it is not the current directory.

The parameter <start_of_flash> is optional, if not present, 0 is assumed for it. This value is required for the programming-protocoll. Therefore if for example the chip-select for the flash is set for the range $20000-$3FFFF, <start_of_flash> must be set to $20000.

But during programming, no offset will be added to the addresses in the S-Record file, therefore the chip-select for the flash must be set to the addresses stored in the S-Record file.

During programming, for each record (this means for each line in the S-Record file) one dot is displayed on the screen.

For 16-Bit flashes, the syntax is the same:

AMDP16 <filename> <start_of_flash>

----------------------------------------------------------------------

Erasing:

Preparation for erasing:

Before erasing can start, the same preparation has to be done as for programming (see above).

Syntax for erasing:

AMDERA8 <start_of_flash>

The parameter <start_of_flash> is optional, if not present, 0 is assumed for it. This value is required for the erase-protocoll. Therefore if for example the chip-select for the flash is set for $20000-$3FFFF, <start_of_flash> must be set to $20000.

For 16-Bit flashes, the syntax is the same:

AMDERA16 <start_of_flash>

----------------------------------------------------------------------

Also the source-codes for this routines are provided to give an example, how the target resident drivers of BD32 are working. (Un)fortunately the comments in the sources are in german language.

It gives also an easy way to change these source files for other flash types.

If you modify the sources for other flashes, please upoad your files again to this mailbox to give some help to other users (as you got help from these programs).

If you are fixing bugs (have you ever seen bugfree software other than yours?) or enhancing functionality then please upload the changed files again.